- PF400 Robot Plug-In for TCS

- Version 2.0A4, April 2012

# Overview

This document describes the PF400 Robot Plug-In for the Precise Automation TCP Command Server (TCS) application.  It contains a number of commands specific to the PF400 robot and its electronic gripper.

To use this plug-in,  load the Tcp_cmd_server_pf400 project which consists of the standard TCS plus the plug-in file PF400.gpl

Within this file, the module PF400 contains commands for the PF400 robot, and the module SSGRIP contains commands for the Precise Spring Servo Gripper.

# PF400 Commands

The table below describes the commands contained in this plug-in.

See the TCP Command Server User's Guide for information about the standard TCS commands.

These commands apply to a PF400 Robot with a Precise Servo and Pneumatic Gripper.

| Command Case ignored | Arguments | Description, responses are for PC mode. Telnet mode is similar but more descriptive. |
| --- | --- | --- |
| movetosafe | none | Moves PF400 Robot to Safe Position. Does not include checks for collision 3rd party inside obstacles inside the work volume of the robot |
| changeconfig | arg1: Grip mode. 2 = close gripper, <> 2 = open gripper. | Change Robot configuration from Righty to Lefty or vice versa. Does not include checks for collision inside work volume of the robot. Can be customized by user for their work cell configuration |
| pickplate | arg1: Station ID, from 1 to 20. Corresponds to TCS locidx value + 1 | Moves to predefined position and picks up plate. If the arm must change configuration, it automatically goes through the Park position. At the conclusion of this routine, the arm is left gripping the plate and moving to the nest approach position. Use Teach function to teach station pick point. |
| placeplate | arg1: Station ID from 1 to 20. Corresponds to TCS locidx value + 1 | Moves to predefined position and place a plate. If the arm must change configuration, it automatically goes through the Park position. At the conclusion of this routine, the arm is left gripping the plate and moving to the nest approach position. Use Teach function to teach station place point. |
| teachplate | arg1: Station ID, from 1 to 20. Corresponds to TCS locidx value + 1 arg2: Optional. The Z Clearance value. If omitted, a value of 50 is use. If specified and non-zero, this value is used. | Sets the plate location to the current robot position and configuration. The location is saved as Cartesian. Z clearance must be high enough to withdraw the gripper. |
| freemode | arg1: Axis selection. -1 = Unfree all axes. 0 = Free all axes. > 0 = Free just this axis. | Activates or deactivates free mode. |
| gripper | arg1: Grip mode. 1 = Open gripper 2 = Close gripper | Opens or closes the servoed gripper. |
| gripopenpos | arg1: If omitted, the gripper open position is returned. If specified, contains the new gripper open position. | Gets or sets the gripper open position for PF400 servo gripper. |
| gripclosepos | arg1: If omitted, the gripper close position is returned. If specified, contains the new gripper close position. | Gets or sets the gripper close position for PF400 servo gripper. |
| StationType | arg1: If omitted, the station type is returned. If specified, contains the new station type. 0 = horizontal, 1 = vertical. | Gets or sets the station type for PF400 robot for the pickplate and placeplate commands. |

# SSGRIP Commands

These commands operate the Precise Spring Servo Gripper for use in lab automation applications.

| Command Case ignored | Arguments | Description, responses are for PC mode. Telnet mode is similar but more descriptive. |
| --- | --- | --- |
| HomeAll_IfNoPlate | none | Tests if the gripper is holding a plate. If not enable robot power and home all robots. Returns -1 if no plate detected and the command succeeded. Returns 0 if a plate was detected. |
| GraspPlate | arg1: Plate width, in mm. Should be accurate to within about 1 mm. arg2: Percent speed to close fingers. 1 to 100. arg3: Maximum gripper squeeze force, in Nt. | Grasps a plate with limited force. The actual closing distance is about 1 mm smaller than arg1 so that the PID servos generate a gripping force. Due to noise, friction, and the spring, actual gripping force may vary somewhat. Returns -1 if the plate has been grasped. Returns 0 if the final gripping force indicates no plate. |
| ReleasePlate | arg1: Open width, in mm. arg2: Percent speed to open fingers. 1 to 100. arg2: Optional. The standard InRange profile property for the gripper open move. If omitted, a zero value is assumed. | Releases the plate after a GraspPlate command. Opens the gripper to the specified width and cancels the force limit once the fingers are open. |
| IsFullyClosed | none | Tests if the gripper is fully closed by checking the end-of-travel sensor. Returns -1 if the gripper is within 2mm of fully closed. Otherwise returns 0. |